NS -95

Version 2.10

Instruction Manual

| Distributed by: |  |  |
| --- | --- | --- |
| Nikon Corporation Surveying Inst. Division Int’l Sales Section 9-16 Ohi 3-chome Shinagawa-ku Tokyo 140-8505, Japan + 81-3-3773-8123 | Nikon Inc. Instrument Group Surveying Department 1300 Walt Whitman Road Melville, New York 11747-3064, U.S.A. + 1-516-547-8500 | Nikon Europe B.V. Instrument Department Schipolweg 321 P.O. Box 222 1170 AE Badhoevedorp The Netherlands + 31-20-449-6222 |

IMPORTANT NOTICE

The information contained in this document is subject to change without notice.

Nikon Corp. makes no warranty of any kind with regard to this material, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose.

Nikon Corp. assumes no responsibility for the use or reliability of its software on equipment that is not approved by Nikon Corp.

This document contains proprietary information which is protected by copyright.  All rights reserved worldwide.  No part of this document may be photocopied, reproduced, or translated to another language without the prior written consent of Nikon Corp.

COPYRIGHT ACKNOWLEDGMENT

Copyright 1997, 1998, 1999

Nikon Corporation

Surveying Inst. Division

9-16 Ohi 3-chome

Shinagawa-ku, Tokyo 140-8505, Japan

REGISTERED TRADEMARKS

IBM PC is a registered trademark of International Business Machines.

HP is a registered trademark of Hewlett-Packard Company

AutoCAD and DXF are registered trademarks of AutoDesk Inc.

All other trademarks are the property of their respective holders.

PRINTING HISTORY

Edition 1	June 1994

Edition 2	Dec. 1995

Edition 3	July 1997

Edition 4	Aug. 1998

Edition 5	June 1999

TABLE OF CONTENTS

Prestart	Introduction					P-1

Before You Begin				P-1

NS-95 Package Contents			P-1

Manual Conventions				P-2

Chapter 1	Installation					1-1

1.1	Installation on a PC			1-1

1.2	Installation on HP-95/100/200	1-2

1.3	Additional info. on HP-95/100/200	1-3

Chapter 2	Getting Started				2-1

2.1	Starting NS-95 from PC		2-1

2.2	Starting NS-95 from HP-95LX	2-1

2.3	Starting NS-95 from HP-100/200LX	2-2

2.4	Initial Startup				2-3

Chapter 3	Main Menu					3-1

3.1	Main Menu Functions			3-2

Chapter 4	Job Manager					4-1

4.1	Screen Areas				4-2

4.2	Highlight Bar				4-2

4.3	Movement Keys			4-3

4.4	Job Manager Functions		4-3

Creating Job			4-3

Creating Subdirectory		4-4

Copying Jobs / Files		4-5

Moving Jobs / Files		4-5

Chapter 4	Job Manager (cont.)

4.4	Job Manager Functions (cont.)

Job Info			4-6

Activating Job			4-6

Deleting Job			4-6

4.5	Function Key Summary Table		4-7

4.6	Job Manager Action Table		4-8

Chapter 5	Add Coordinates				5-1

5.1	Movement Keys			5-1

5.2	Input Fields				5-2

PT. #				5-2

PT. ID.			5-2

Northing / Y			5-3

Easting / X			5-3

Elevation / Z			5-3

Code / Description		5-3

Chapter 6	Import Data					6-1

6.1	Selecting the Type of Data		6-1

6.2	ASCII Coordinate File		6-2

6.3	Civilsoft File				6-5

Chapter 7	Exporting Data				7-1

7.1	ASCII Coordinate File		7-2

7.2	Nikon RAW File			7-2

7.3	Nikon RAW V2.00 File		7-3

7.4	AutoCAD DXF File			7-3

7.5	Tripod Data Systems File		7-4

7.6	Sokkia SDR-2x File			7-4

7.7	Civilsoft File				7-5

7.8	Wild Topo File			7-5

7.9	SoftDesk File				7-5

Chapter 8	Settings					8-1

Chapter 9	Code & Parameter Files			9-1

Appendix A	Command Line Operation			A-1

Introduction

This guide describes the operation of Nikon's NS-95 Database Utility.

NS-95 is a utility program for the Nikon DTM-800 Field Station and Nikon AP-800 Field Station Software.

The primary function of NS-95 is to allow users all AP-800 survey data management ability without the use of the Nikon DTM-800 Field Station.  NS-95 is designed to run on a standard MS-DOS personal computer.  In addition, NS-95 will run on the Hewlett Packard HP-95LX and HP-100/200LX Palmtop Computers.

Specific features of NS-95 include the ability to create, move, copy, and delete jobs.  Coordinate records may be manually input or imported from an ASCII coordinate file. Existing survey data can be exported to a number of popular data file formats. NS-95 can be used to create binary code and parameter files.

Before You Begin

NS-95 Package Contents

Your NS-95 package (3.5" disks) contains the following files:

·	NS-95 Program files

·	NS-95 Instruction Manual in Microsoft Word (DOC)

·	NS-95 Instruction Manual in ASCII text (TXT)

·	Readme.txt file

·	NIKON.COD and NIKON.PAR files and their original ASCII files
(Examples of Code and Parameter files)

Manual Conventions

This manual uses the following conventions:

Typefaces	Description

Main Menu	Indicates menu options, screens and

functions.

[ENTER]	Indicates a keyboard key to be pressed

to initiate an immediate action.

[DEL], [J], [E] are examples.

text		Indicates alpha or numeric text entry by

the user.  Generally followed by one of

the above action keys.

- Notes contain important information which is set off from the text.

- Warning messages appear before procedures which, if not observed, could result in some problem for the user.

AP-800 Familiarity

As the NS-95 software is a utility for use in conjunction with Nikon's DTM-800 Field Station and AP-800 Field Station Software, some familiarity with the DTM-800 System is assumed.  For questions regarding Jobs, File Formats, and Data Cards, refer to the Nikon AP-800 Reference Manual.

Installation

NS-95 is a small utility program distributed on a single 3.5" floppy disk.  NS-95 will run on the HP-95/100/200LX Palmtop Computers and  a standard MS-DOS PC.  Select the installation procedure for your PC.

1.1	Installing NS-95 on a PC.

·	Power on PC.

·	Exit any DOS shell such as MS-Windows to the standard DOS prompt.

·	Type CD\ and press [ENTER] to return to the root directory.

·	Make a directory on the PC in which NS-95 will reside.

·	Type MD NS95 and press [ENTER]

·	Change to the newly created directory.

·	Type CD NS95 and press [ENTER]

·	Insert the NS-95 Disk into 3.5" disk drive.

·	Type COPY A:*.* and press [ENTER]

The NS-95 software is copied to the NS-95 subdirectory on your PC.

The installation is now complete.  To begin using NS-95, proceed to Chapter 2, "Getting Started."

1.2	Installing NS-95 on a HP-95/100/200.

Depending on how much data already exists on the HP-95/100/200, NS-95 will probably not fit on the RAM disk of the HP.  In this case, NS-95 can be run directly from a RAM memory card such as the DTM-800 data card (512kb and larger only).  For this process, it is necessary to use a PCMCIA card drive such as the Databook or Data I/O.  It is also necessary to know the drive designation letter that is assigned to that drive.  In most cases it is D: but may be any letter above C:.  For installation purposes, it will be called D:.

·	Power on PC.

·	Exit any DOS shell such as MS-Windows to the standard DOS prompt.

·	Insert RAM data card into PCMCIA card drive.

·	Type D: and press [ENTER] to change to the card drive.

·	Type CD\ and press [ENTER] to change to the Root Directory of D:

·	Insert the NS-95 Disk into 3.5" disk drive.

·	Type COPY A:*.* and press [ENTER]

The NS-95 software is copied to the RAM data card.  NS-95 can be run directly from the card by the HP palmtop computer.

To begin using NS-95, proceed to Chapter 2, "Getting Started."  For additional HP installation instructions proceed to the next section.

1.3	Additional Installation on HP-95/100/200

If NS-95 is to be copied to the RAM disk of the HP-95/100/200, the following additional procedure may be used.

·	Insert the RAM data card into the card drive of the HP-95/100/200.

·	Select the [FILER] application key on the keyboard of the HP-95/100/200.

·	Select the [GOTO] softkey on the HP-95/100/200.

·	Type A: and press [ENTER].

·	Move the highlight bar to the file named NS95.EXE and press the [COPY] softkey.

·	Type C:\ and press [ENTER].

The program is copied to the RAM drive of the HP-95/100/200 provided there is enough room on the RAM drive.  If there is not enough room, the HP will display an error and abort the operation.  In this case, NS-95 must be run from the RAM data card.  To begin using NS-95, proceed to Chapter 2, "Getting Started".

- NS-95 is a rather large program for the HP Palmtop Computers.  Having the program reside on the AP-800 data card will greatly reduce the amount of field survey data that can be stored on that data card.

Getting Started

The NS-95 software runs exactly the same independent of the computer it is run on.  How the program is started differs somewhat depending on which type of computer is being used.  In this section, the method for starting NS-95 from a regular PC and from the HP Palmtop Computer will be described separately.

2.1	Starting NS-95 from a PC.

·	Exit any DOS shell software such as MS Windows.

·	Return to the root directory by typing CD\ and pressing [ENTER].

·	At the C:\ prompt, type CD\NS95 and press [ENTER].

·	Type NS95 and press [ENTER].

2.2	Starting NS-95 from a HP-95LX.

·	Exit all currently running applications (Phone, Memo, Lotus, etc.) by pressing [MENU] then [Q] until applications have been exited and the Main Menu is displayed

·	Select the [FILER] application key from the keyboard of the HP.

·	If the NS-95 is located on the data card, select the drive where NS-95 is located by pressing [F5] [Goto] and typing A: [ENTER].

·	Use the arrow keys on the HP to move the highlight bar to the NS95.EXE file.

·	Press the [F4] [Run] softkey to start NS-95.

2.3	Starting NS-95 from a HP-100/200LX.

·	Exit all currently running applications (Phone, Memo, Lotus, etc.) by pressing [MENU] then [Q] until applications have been exited and the Main Menu is displayed

·	Select the [FILER] application key from the keyboard of the HP.

·	If NS-95 is not located on the current drive, select the drive where NS-95 is located by moving the highlight bar to the "View Up" selection located at the top of the directory.  Press [ENTER] to display the available drives.  If NS-95 is located on a data card, move the highlight bar to the [-A-] drive selection and press [ENTER].

·	Use the arrow keys on the HP to move the highlight bar to the NS95.EXE file.

·	Press [ENTER] to start NS-95.

2.4	Initial Startup.

When NS-95 is run for the very first time, the Settings Menu is displayed before the Main Menu.  This is so the user may define the units to be used and the way in which the software will work.

Fig. 2.1: Settings Menu Screen 1

Each Setting Option is selected by pressing the letter of the bracketed key of the desired option.  Each time the key is pressed, the option will change showing the next available option.

Most of these options are duplicates of options in the AP-800 software.  It is important for the settings in NS-95 to match the comparable settings in AP-800.

Once the proper settings are selected from this menu, press [N] to display the Next setting screen.  After all setting are correct, press [S] to Save the settings.  Then press [ESC] to return to the Main Menu.

For more detailed information on SETTINGS, refer to Chapter 8 of this manual and Chapter 13 of the AP-800 Reference Manual.

The Main Menu

NS-95 displays a small main menu containing six options.  These options are discussed in the following chapters.  Each option is selected by pressing the key that corresponds to the first letter of the option.  This letter is displayed in brackets.

The currently active Job is displayed in the lower left corner of the screen.  If no job is currently active (as when NS-95 first starts), the words NO JOB appear on the screen.

Fig 3.1: NS-95 Main Menu

On a PC screen the Main Menu appears as a small box in the center of the screen.  This small screen size is to accommodate the smaller screen size of the HP-95LX Palmtop Computer.  Although the screen size is small on a PC, it is full size on the HP-95LX.

The user may exit NS-95 at any time by pressing [ESC].  The user will be returned to DOS after exit.  The [ESC] key is the all purpose exit key used in NS-95.  Pressing [ESC] in any screen at any time will abort an operation and return the user to the previous screen in the program.

3.1	Main Menu Functions

Below is a brief description of each menu item in the NS-95 Main Menu.

[J]ob Manager

·	Create a new AP-800/700 Survey Job.

·	Create a new DOS Sub-Directory.

·	Delete an existing AP-800/700 Survey Job / File / Directory.

·	Copy AP-800 Survey Job / DOS File.

·	Move AP-800 Survey Job / DOS File.

·	Change Drive / Directory.

·	View AP-800/700 Job Info.

See Chapter 4 Job Manager.

[A]dd Coordinates

·	Manually input coordinate records.

See Chapter 5 Add Coordinates.

[I]mport File Data

·	Import ASCII coordinate text file.

·	Import Civilsoft file.

See Chapter 6 Import Data

[E]xport File Data

·	ASCII coordinate file.

·	Nikon Raw format file.

·	Nikon Raw format V2.00 file.

·	AutoCAD DXF file.

·	Sokkia SDR-2x file.

·	TDS Raw Data file.

·	Civilsoft file.

·	Wild Topo file.

·	SoftDesk file.

See Chapter 7 Export Data

[S]ettings

·	Define NS-95 operating parameters.

See Chapter 8 Settings

[C]ode & Parameter Files

·	Convert code and parameter files from ASCII text to binary format.

See Chapter 9 Code & Parameter Files

[J]ob Manager

The Job Manager is the heart of the NS-95 software.  The primary function of the Job Manager is to create and activate (open) AP-800 Survey Jobs right on the DTM-800 Data Card without using the instrument.  Additionally, the user may delete, move, and copy AP-800 jobs, regular DOS files and subdirectories.

To select the Job Manager press [J] at the Main Menu.  The job manager screen shows the contents of the current directory.

Fig 4.1:	 Job Manager

Directory Header

Directory Area

Function Key Bar

- Although NS95 v2.x and AP800 can work with AP700 and AP800 jobs, AP700 (on DTM-700 series) will not be able to read jobs created for AP800.

4.1	Screen Areas

1.	The directory header is at the top of the window.  This contains the column labels for the job manager.  The Job Manager shows the job or filename, the size of the job or file, and the date and time it was created.

2.	The area below the directory header is the directory area.  Here, all of the parent directory, subdirectory, job and file names are displayed.

3.	The function key bar is located across the bottom of the window.  The available function keys are listed from left to right.  Each of these labels corresponds to the function key of the same position.  Since NS-95 is designed to run on palmtop computers, the function key label will not line up with the actual key as in some PC software.  These function keys are discussed in more detail shortly.

4.2 	Highlight Bar

When the Job Manager begins, the first item in the directory area is highlighted.  This highlight indicates the directory item on which some action will be taken by the user.  It is identical to the highlight bar in the Job Manager in AP-800.  Use the up and down arrow keys to move the highlight one item at a time.  The page up and page down keys move an entire screen at a time if there are more than twelve directory entries.  The files are shown in order of subdirectories first, then AP-800 Job files followed by all the other types of files listed in alphabetical order.

- If NS-95 is started from a subdirectory, the first item below the header line is two dots followed by the label  <Parent Dir>.  This indicates that you are in a subdirectory.

4.3	Movement Keys

The following keys are used to move the highlight bar through the currently displayed directory.

| Key | Movement |
| --- | --- |
|  | Moves highlight bar up one item. |
|  | Moves highlight bar down one item. |
| [PG Up] | Moves highlight bar up one screen at a time if more than twelve items. |
| [PG Down] | Moves highlight bar down one screen at a time if more than twelve items. |
| [HOME] | Moves highlight bar to first directory entry. |
| [END] | Moves highlight bar to last directory entry. |

Fig. 4.2:	Movement Keys

4.4 	Job Manager Functions

The following keys perform an action on the currently highlighted item in the directory.

[F1] Create	Allows an AP-800 Job or DOS subdirectory to be created.  The user will be prompted for the type of item to be created.  This option may be selected at any time regardless of what item is currently highlighted in the Job Manager.

Creating Job

Pressing [A] for AP-800 Job will display a standard AP-800 job creation screen similar to creating a job using AP-800.

Fig 4.3:	Job Creation Screen

The job creation screen consists of four input fields.  The Job Name field must be filled out completely.  The job name must be no more than eight characters long and must not contain a file extension.  The job name may include a complete drive and path if the user wishes to create the job on some other drive and/or directory.  The description, client, and comment fields are optional.

After entering information in a field, pressing [ENTER] or [Dn Arrow] will move the user to the next field.  If a mistake is made in input, use the Arrow and [DEL] keys to move between fields and make corrections.  Pressing [ENTER] or [Dn Arrow] on the last field, will accept all information, create the job, make it the currently active job, and return the user to the Job Manager.

Creating Subdirectory

To create a subdirectory select [D].  The user is prompted for the name of the subdirectory to be created.  The name of this subdirectory must conform to standard DOS directory names.  The filename may include another drive and path if you do not wish the job to be created in the current directory.

[F2] Copy	Allows the highlighted item (job, file, subdirectory) to be copied to another drive and/or directory.  After selecting Copy, the user is prompted for the destination of the item to be copied.  The destination must be a complete DOS path including drive path and filename. No extension is used when copying jobs.

Ex:	C:\SUBDIR\Filename.ext

The filename may be different than the current name if the user wishes to change the name.  Once completed, the original file will remain in the current directory with the copy in the destination directory.  The user may not copy a file into the same directory unless the name of the file is changed.  If an invalid drive, path, or filename is entered, the message "Invalid Filename" will be displayed.  Pressing [ESC] will clear the error message and return the user to the Job Manager.

[F3] Move	Allows the highlighted item (job, file, subdirectory) to be moved to another drive and/or directory.  After selecting Move, the user is prompted for the destination of the item to be moved.  The destination must be a complete DOS path including drive, path, and filename.  Unlike [F2] Copy, a copy of the highlighted item will no longer remain in the current directory.  If an invalid drive, path, or filename is entered, the message "Invalid Name" will be displayed.  Pressing any key will clear the error message and return the user to the Job Manager.

[F4] Drive	Allows the user to select a different disk drive for reading by the Job Manager.  Once selected, the user may input the drive letter of the drive to be viewed by simply inputting the letter and pressing [ENTER].  This option may be selected at any time, regardless of which item is currently highlighted.  Only the letter of active disk drives are allowed.  If a non-active drive letter is input, the message "Invalid Drive" will be displayed.  Pressing any key will clear this message and return the user to the Job Manager.

[F5] Info	Shows the currently highlighted job information that was input at the time the job was created.  This operation will only function on an AP-800 Job.  Selecting this function while any other type of file is highlighted will display "Not Supported File Type" message.  Pressing any key will clear this message and return the user to the Job Manager screen.

[ENTER]	The primary function of this key is to open an AP-800 Job and make it the currently active job for other NS-95 functions. In addition, if [ENTER] is pressed while a subdirectory or parent directory is highlighted, NS-95 will change to that directory.

[DEL]		Pressing the [DEL] key on any highlighted AP-800 job, DOS file, or Subdirectory will delete that item.  As a safety measure, the user is prompted, "Delete Name.XXX?? (Y/N)".  The user must press [Y] for Yes to delete this job, file or directory. To cancel the operation, press [N] for No.   Once a file is deleted, NS-95 provides no method for recovering the file.  Later versions of DOS provide the ability to recover a deleted file provided nothing else has been written to the disk / card.  Refer to the DOS manual for recovering a deleted file.

A subdirectory must be empty (i.e. not contain any files) before it can be deleted.  If the subdirectory is not empty, the message, "Permission Denied" will be displayed.  This message will also display if the directory is a "Read Only" directory.  ("Read Only" files and directories are beyond the scope of this software and manual. See the MS-DOS manual for more information.)

4.5 	Function Key Summary Table

| Key | Function |
| --- | --- |
| [F1] Create | Create an AP-800 Job, AP-700 Job or subdirectory. |
| [F2] Copy | Copies the highlighted job or file to a user specified name or place |
| [F3] Move | Moves the highlighted job or file to a user specified name or place |
| [F4] Drive | Changes to a different drive. |
| [F5] Info | Shows the job information on the highlighted AP-800/700 Job. Has no effect on any other type of file. |
| [ENTER] | Opens AP-800/700 job and makes it the currently active Job. Changes to highlighted subdirectory. Changes to highlighted parent directory |
| [INS] | Create an AP-800 Job or subdirectory. |
| [DEL] | Deletes highlighted AP-800/700 Job. Deletes highlighted DOS file. Deletes empty directory. |

4.6 	Job Manager action listed by highlighted item.

| Highlighted Item | Operation | Key |
| --- | --- | --- |
| Parent Directory | Move to parent directory. | [ENTER] |
| Subdirectory | Create subdirectory.* Copy subdirectory. Move subdirectory. Enter a subdirectory. Delete subdirectory. | [F1] Create [F2] Copy [F3] Move [ENTER] [DEL] |
| AP-700/800 Job | Create new Job.* Delete Job. Copy Job. Move Job. View Job Info. | [F1] Create [DEL] [F2] Copy [F3] Move [F5] Info |
| DOS File | Delete DOS File. Copy DOS File. Move DOS File. | [DEL] [F2]Copy [F3] Move |

- * Operation can be done at any time 	regardless of what directory item is highlighted.

[A]dd Coordinates

The Add Coordinates option is used to input coordinate records directly into the currently active job.  A coordinate record includes a point number, point ID (optional), Y or Northing, X or Easting, Elevation, and Code or Description.  These coordinate records may then be used by the AP-800 software for Station Orientation, Resection, Stakeout, or COGO.

To select Add Coordinates, press [A] at the Main Menu.

Fig. 5.1:	Add Coordinate Screen

5.1 	Movement Keys

The Add Coordinate Record screen displays 6 input fields.  The user may move to the next field by pressing Down Arrow or ENTER.  The user may move to the previous field by pressing Up Arrow.  Pressing [ENTER] or Down Arrow on the last field will cause NS-95 to check for errors and add the record to the job if no errors are found.

The left and right arrow keys, back space, and [DEL] keys are used to edit the individual fields.  Left and right arrow keys are used to move the cursor left and right one character at a time.  The back space key will delete one character to the left of the current cursor position.  The [DEL] key will delete one character to the right.  These four keys used in combination will allow the data in a field to be deleted as necessary.  Typing in new characters will overtype the character in the current cursor position.

5.2 	Input Fields.

PT. #.		This is the unique point number that the user assigns to the coordinate values entered.  The point number must be an integer in the range 1-9999999.   Upon entry of the current record, this point number is automatically incremented by 1 and displayed in the next coordinate record input screen.  No alpha characters are allowed in the point number field.

PT. ID.	Point ID is an optional field that can be used to further identify a point by an alphanumeric name.  Using AP-800, points can be searched by this ID in addition to using a point number.  For example: GPS MON 2.  This field has a maximum of 10 characters that may be any combination of alphanumerics the user desires.

- Certain symbol characters available on a standard PC keyboards are not available on the DTM-800 keyboard.  If certain symbols are used in the PT. ID field (EX: @,!%{}|/"),  it will not be possible to search for the point by this field in the AP-800 software.

Northing / Y	The Northing or Y value is required in the coordinate record.  This value may be any real value of fourteen digits or less.  (Ex: -999999999.999 to 9999999999.999)  No alpha characters are allowed in the Northing.

Easting / X   	The Easting or X value is required in the coordinate record.  This value may be any real value of fourteen digits or less.  (Ex: -999999999.999 to 9999999999.999) No alpha characters are allowed in the Easting.

Elevation / Z	The Elevation or Z value is required in the coordinate record. This value may be any real value of fourteen digits or less.  (Ex: -999999999.999 to 9999999999.999).  No alpha characters are allowed in the elevation.

Code		Also known as a description, this is an optional field that may contain any alphanumeric string up to 30 characters long.  The user may search by this field in AP-800 for stakeout purposes.

Pressing [ENTER] or Down Arrow on the code field will accept the point into the job provided there are no syntax errors in the data.

- The labels and order of the input fields depend on the current settings of NS-95.

If invalid data is in the record, NS-95 will display "Invalid Record" message.  Pressing any key will clear this message and return the user to the PT # field of that record allowing the error to be corrected by the user.

If a record with a duplicate point number is entered, NS-95 will display the message "Duplicate Records Found" and the user will be returned to the Add Coordinate Record screen.  The user may change the point number and accept the remaining data for input.

To exit Add Coordinate Record, press the [ESC] key at any time.  The user will be returned to the Main Menu.

[I]mport Data

One of  the most useful features of NS-95 is the ability to import ASCII coordinate data.  Most third party surveying software manufacturers offer the ability to create a comma delimited ASCII coordinate file.  This file can be imported into an AP-800 job using this feature.

Another import format supported by NS-95 is the Civilsoft coordinate data file format. This file can be imported into an AP-800 job using this feature.

After selecting the file to be uploaded, NS-95 will begin importing the file records into the currently active job.  The progress of the import is displayed in the record counter on the screen.  The import may be aborted at any time by pressing the [ESC] key.

6.1	Select the Type of Data

To import an ASCII coordinate file or a Civilsoft file, press [I] at the Main Menu.   The Import File Type screen will appear.

Fig. 6.1:	Import File Type

To select the ASCII Coordinate File format, press [A]. To select a Civilsoft format, press [C].

6.2	ASCII Coordinate File

NS-95 displays the following screen that prompts you to select the format for the data fields in the imported data.

Fig. 6.2: Selecting the Data Fields to Import

The user may choose import options from the following list in any order:

- Point Data (Pt)*

- Identifier (ID)

- Northing (N)*

- Easting (E)*

- Elevation (Z)

- Point Code (Code)

- No selection (Blank)

- * The selection must contain data for Pt, N and E or Pt and Z as a minimum.

When the user makes no selection in a field (i.e. sets it to blank), NS-95 ignores the data at that point in the imported data file.

6.2.1	Selecting options and moving between the fields

To select the options in the fields, press the [N] key to scroll through the options until the desired option appears. To move to the next field, press [TAB], or the arrow keys.

6.2.2	Saving the data format selections

The user can save the format for the selected data fields so that the selection is remembered by NS-95. To do this, make the selections as described above and press [S] (save).

6.2.3	Accepting the selections and importing data

When the selections are complete, put the cursor in the last field on the screen and press [ENTER].

The Import ASCII Coordinates screen will appear. The user is prompted for the name of the file to be imported.  This filename may include the complete drive and path if the file is located somewhere other than the current working directory.

Fig. 6.3: Import ASCII Coordinates

6.2.4	Duplicate Records

When NS-95 encounters a duplicate record, a warning screen appears showing the record number that is duplicated in the imported file.

Fig. 6.4: Duplicate Record Warning Message

The options in this screen are as follows:

- ESC ENDS - stops importing data at this point and displays an ‘Operation Cancelled’ screen.

- [O]VERWRITE - overwrites the existing record with the data from the imported file.

- [S]KIP - ignores this record from the imported file and continues to import the data.

- [A]LL - overwrites this record and all subsequent duplicates.

- A duplicate record is defined as a record that has the same point number as a point already in the job, regardless of what any of the coordinate values or codes are.

Choose the required option and follow the prompts on the screen. When the data is imported, NS-95 displays the completion screen and returns the user to the main menu.

6.3	Civilsoft File

The Import Civilsoft Coordinates screen prompts the user for the name of the file to be imported.  This filename may include the complete drive and path if the file is located somewhere other than the current working directory.

Fig. 6.5: Import Civilsoft Coordinates

[E]xport Data

Users may wish to use other third party surveying software for data processing.  NS-95 has the capability of exporting AP-800 data to other popular file formats.  These other data formats can then be read by the third party software.

To select Export File Data, press [E] at the Main Menu.

Fig. 7.1:	Export File Menu

The following file formats are available for Export:

·	ASCII coordinate file (comma delimited)

·	Nikon RAW data file.

·	Nikon RAW data V2.0 file.

·	AutoCAD DXF file.

·	Tripod Data Systems (TDS) file.

·	Sokkia SDR-2x file.

·	Civilsoft file.

·	Wild Topo file.

·	SoftDesk file.

Select the file format by pressing the letter in brackets of the format desired.  After selecting the desired format for Export, NS-95 will ask the user for the name of the file to be created.  The filename may include the complete path if the file is to be located somewhere other than the current working directory.  Upon input of filename, NS-95 will convert the data from the currently active job to the  newly named file in the selected format.

Progress of the data export is displayed on the record counter on the screen.  The export operation may be canceled at any time by pressing [ESC].

After Export completion or aborting, the user is returned to the Main Menu.

7.1 	ASCII Coordinate File

Almost all popular surveying and civil engineering software have the capability of reading a comma delimited ASCII (pronounced AS-KEY) coordinate file.

The file produced by NS-95 contains one ASCII coordinate record for every AP-800 coordinate or measurement in the job.  Job comments and deleted records are not transferred.  Each record of the ASCII file is in the following format:

Pt Number, Northing (Y) ,Easting (X),Elevation (Z),Code

1234,1000.0000,2000.0000,534.9873,SW1

7.2	Nikon RAW Data File

The Nikon RAW Data File generates an ASCII text file identical to that of Nikon AP-700 Software.  This file format is described in the Appendix of the AP-700 reference manual.  This file contains many different record types and record formats.

  - Nikon RAW Data V2.00 File

The Nikon RAW Data V2.00 File generates an ASCII text file identical to that of Nikon AP-800 Software.  This file format is described in Appendix A-14 of the AP-800 reference manual.  This file contains many different record types and record formats.

7.4	AutoCAD DXF File

When selected, this option will generate a DXF (Drawing X-change Format) file that is compatible with many different types of CAD software including AutoCAD, TurboCAD and others.

The DXF file that is created is only a point file. Once imported to a CAD package, the data will appear as dots on the screen with the point number and optional elevation next to each dot.  Each dot will be placed at the exact Northing (Y) and Easting (X) of the coordinate value.

When creating a DXF file there are two parameters the user must defined after inputting the target filename.  The first is whether or not memos are to be included in the transfer.  Select [Y] for Yes or [N] for No.  The second is the height of the text desired in the transfer.  Only integer values are accepted in this field and the default is one (1).  The units of the text height are defined by the current units of the AutoCAD drawing importing the DXF file.

Each item of the point is on a different layer of the drawing so that any of the features about the point may be turned off or on as a layer.  The dot that is the actual point location in the drawing is on the layer named PT.  The point number is on a layer named PN.  The elevation of the point is on the layer named EL and the Code / Description is on the layer named DE.  For a complete description of Layers, refer to your CAD manual.

7.5	Tripod Data Systems File Format

Another format available from NS-95 is the Tripod Data Systems file format.

This format is particularly useful when the destination for the AP-800 data is an Intergraph Workstation.  Tripod Data Systems is a supplier of Intergraph Workstation Interfaces.  By creating a file of this format and using the Tripod Data Systems interface, it is very easy to convert AP-800 raw field data into an Intergraph Workstation file.

The complete description of this file format can be found in any TDS data collector manual or can be obtained by contacting TDS.

7.6	Sokkia SDR-2x File Format

Another popular method of importing data into various software packages is via a Sokkia SDR-2x data file.  Most surveying software packages have the ability to read a SDR data file.  The advantage of this file structure is the actual unadjusted raw field data is sent to the software.  This method is generally more desirable for many users because it allows a traverse to be adjusted or a rod height to be edited.

This is the same file format available from the Nikon AP-800 Software in the Download portion of the Communications section.

For the exact definition of the file format, refer to the Sokkia SDR Manual.  An example of the file can be found in Appendix A-21 of the Nikon AP-800 Reference Manual.

7.7	Civilsoft File Format

NS-95 exports coordinate data in Civilsoft input format.

The complete description of this file format can be found by contacting Civilsoft.

7.8	Wild Topo File Format

Another format available for export by NS-95 is Leica’s (formerly Wild) Wild Topo format.

The complete description of this file format can be found by contacting Leica.

7.9	SoftDesk File Format

SoftDesk File Format is also supported by NS-95.  Similar to Sokkia’s SDR-2x file format, SoftDesk’s file format contains actual unadjusted raw field data.

Parameters may be used in the code field during data collection for importing strings into SoftDesk.

The supported parameters are found in the following table:

| Parameter | Definition |
| --- | --- |
| B | Begin Polyline |
| ST | Start of Polyline (insert at beginning of polyline) |
| C3 | 3 point curve |
| C | Continue |
| PC | Point of curvature |
| PRC | Point of reverse curvature |
| PT | Point of tangency |
| E | End of Polyline |

These parameters must be positioned before the string name in the code field.

A typical case could be as follows:

B CURB

CURB

MH

CB

PIN

C CURB

CURB

E CURB

For a complete description of the SoftDesk FieldBook format, please contact SoftDesk.

[S]ettings

NS-95 has numerous options allowing the software to operate the way different users prefer.

When NS-95 is run for the first time, this settings screen is displayed so the user may set the options.

There are two screens for the settings options.  Pressing [N]ext will toggle between the first and second setting screens.  Pressing [S]ave will save the current settings.  Pressing [ESC] will exit the Settings Menu.  Any changes made in the Settings but not saved will not be in effect the next time NS-95 is run.

Fig. 8.1:	NS-95 Settings Menu Screen 1

Fig. 8.2:	NS-95 Settings Menu Screen 2

The following charts shows the settings and the available options for each setting.  To obtain identical data between NS-95 and AP-800/700, the settings in NS-95 must match the comparable settings in AP-800/700.

8.1 Settings Charts

| Setting (Screen 1) | Option | Use |
| --- | --- | --- |
| Angle Unit | DDDMMSS Dec.Deg. Gons Mil 6400 Mil 6000 | Deg, Min, Sec Decimal Degrees 1 gon = 360 / 400 deg 1 mil = 360 / 6400 deg 1 mil = 360 / 6000 deg |
| Temp. Unit | Fahrenheit Centigrade | Fahrenheit Centigrade / Celsius |
| Pressure Unit | inHg mmHg hPa | Inches Mercury Millimeters Mercury Hectopascals (millibars) |
| Distance Unit | Meters USFt IntFt | Metric US Survey Foot International Foot |
| VA 0 Setting | Zenith Horizon | Zenith = 0 Horizon=0 |
| Coord order | NE / YX EN / XY | Northing / Y first Easting / X first |
| HA Raw Data | HA 0 to BS Azimuth | Backsight = 0 Use Azimuth |

Fig. 8.3: Settings Chart for Setting Screen 1

| Setting (Screen 2) | Option | Use |
| --- | --- | --- |
| Sea Level | On Off | Adjust horizontal distances to sea level |
| C&R | On Off | Adjust horizontal distance for earth curvature and refraction. |
| C& R Factor | User defined | Earth curvature constant. (0.132) is default. |
| Scale Factor | User defined | Adjust horizontal distances for map projection. (1.0 is default. No adjust.) |
| Column Setting | 80 Column Mode 40 Column Mode | PC / HP-100 PC / HP-100 / HP-95* |
| Color Mode | B/W Color MONO | LCD / Monochrome PC / Color HP-95 only* |
| Coord titles | NEZ XYZ | North, East, Elev X, Y, Elev |

Fig. 8.4: Settings Chart for Setting Screen 2

- * Due to the non-standard display of the HP-95LX, the Column Setting is fixed at 40 column and the Color Mode setting has a special mode that is fixed to MONO.  These options cannot be changed when running NS-95 on the HP-95LX.

[C]ode & Parameter Files

The code and parameter files contain data that can be added to record information and later used to categorize the records.

Code files contain many useful codes under the sublists; Survey Marks, Surface Features, Vegetation, Structures, Roads, Railroads, Utilities and General Codes. Parameter files include parameter choices such as Curve Start, Curve End, Start New String, etc.

These files can be created as text files using any conventional text editor (e.g. Notepad) but must be converted to binary files in order to be used with AP800. For more information about creating these files, refer to the AP800 User’s Manual.

NS-95 provides the functions to convert a code file or a parameter file into the binary format using the [C] option on the main menu.

The user is prompted to enter the name of the file that is to be converted, for example if the codes file was named “mycodes.txt”, the user types: mycodes.txt [ENTER] in the field on the screen.

Fig. 9.1: Enter the ASCII File Name to Convert

When the user presses [ENTER], NS-95 examines the text file to determine its type then displays an input screen for the binary file’s name.

- If the file is a parameter file, NS-95 suggests a name for the binary file and adds a .PAR file extension to the name.

- If the file is not a parameter file, NS-95 assumes that it is a codes file and suggests a name with a .COD file extension (see below).

Fig. 9.2: Accept the Filename or Type a New One

- If the file does not exist, NS-95 displays an error message.

To accept the filename for the binary file, press [ENTER] otherwise, type a different name for the file.

- Do not use reserved characters, such as ‘*’ and ‘\’ in the destination filename. Using these characters generates an error. Refer to the MS-DOS user documentation for more information on reserved characters.

NS-95 converts the text file to a binary file that you can copy to the “\CODES” directory on the datacard only when the following conditions are met:

- The file’s structure is valid.

If the text file’s structure is not valid, NS-95 displays an error message. To view the errors, use a text editor to open the NS95-ERR.TXT file. This file provides information, by line number, about the errors in the text file.

- If the text file has the correct structure but has invalid text content, NS-95 will not display an error message. Make sure that the contents of the file are valid for the application.

- The destination binary file does not already exist.

If the binary file exists, NS-95 displays the following message.

Fig. 9.3: File Already Exists Error

To convert the text file, the destination filename must not exist in the destination directory. Choose another filename or save the file to a different location.

Command Line Operation

NS95 has two modes. Previous chapters discuss only the interactive mode which uses menus, browsers, and dialog boxes to interact with the user. It is invoked by typing NS95 at the DOS command line with NO command line arguments.

Command Line Mode uses command line arguments to accomplish database interaction. It provides accessibility to the AP-800 database from third party software (by swapping), as well as noninteractive operation from the DOS command line or a DOS batch file. Command Line Mode returns an exit code to indicate execution status.

The command line format is:	NS95 /switch <ARG1> <ARGx>.

Where switch is the string to specify operation.

ARG1 - ARGx are the parameters required by the operation.

Maximum command line length is 128 characters including

program name (NS95), so make sure that this limit is not exceeded when using NS95 in Command Line Mode.

Available switches are:

/jc7		Create an AP700 job.

/jcr 		Create an AP800 job.

/jcp		Job Copy.

/jmv		Job Move.

/jdl		Job Deletion.

/jad		Append ASCII Coordinate Record to database.

/res		Create or Modify settings file.

/ior		Specifies the types of field to import and their import order.

/iac		Import ASCII coordinate records from file.

/ics		Import Civilsoft coordinate records from file.

/eac		Export all records to file in ASCII coordinate format.

/edx 		Export all records to file in AutoCAD DXF format.

Available Switches (cont.)

/en7		Export all records in the original Nikon Raw (AP700) data.

/enr		Export all records to file in Nikon Raw v2.00 (AP800) data.

/esd		Export all records to file in SOKKIA SDR2x format.

/etd		Export all records to file in Tripod Data Systems TDS format.

/eso		Export all records to file in Softdesk Field Book format.

/ecs		Export all records to file in Civilsoft coordinate format.

/ewt		Export all records to file in Leica TOPOS format.

/cod		Create a binary code or parameter file.

Job Creation:

Ex:	ns95 /jcr jobname “job desc” “job client” “job comment”

jobname:	Name of Job to create. Can include path.

jobdesc:	Description of Job.

jobclient:	Client job is being created for.

jobcomment:	Comments

The asterisk (*) can be used as a place holder for the Job Description, Job Client and Job Comment if any of these fields are not used.  If none of these three fields are used, they can be left blank. Quotes (“) are used to enclose fields that contain spaces but are not required if the field is only one word.

Job Copy:

Ex:	ns95 /jcp fromjob tojob

fromjob:	Job to copy from. Path can be included

tojob:		Job to copy to. Cannot exist before operation

Job Move:       (Rename)

Ex:	ns95 /jmv fromjob tojob

fromjob:	Name of job to move or rename. Path can be 			included.

tojob:		Target Job (or new name). Cannot exist before operation.

Job Delete:

Ex:	ns95 /jdl jobname

jobname:	Name of job to delete. Does not ask for 				confirmation!!

Add coordinate record:

Ex:	ns95 /jad jobname ptnum id north east elevation code

jobname:	Name of job to add coordinate record to.

ptnum:		Point Number, a unique integer value.

id:		Point reference id, another name for this point

north:		Point northing.

east: 		Point easting.

elevation:	Point elevation.

code:		Code or short comment.

All fields must be occupied with a value.  Ptnum must be a unique integer value. ID may be a nine character string without spaces.  North, East, and Elevation must be a floating point or integer value. An asterisk (*) may be used as a place holder if no ID is used and will make the record a type MP.  If a valid ID is used the record will be a type MC.

Create or modify settings file:

Ex: ns95 /res dmode amode hz coord tmode pmode sl cr scale crc HAraw

dmode:Integer designation of the distance unit:

0 -> Metres.

1 -> US feet.

2 -> International feet.

amode:	Integer designation of the angle unit:

0 -> Degrees, decimal.

1 -> Degrees, Minutes, Seconds.

2 -> Gons (400 per 360 degrees).

3 -> Mils (6400 per 360 degrees).

4 -> Mils (6000 per 360 degrees).

hz:	Vertical angle Zero setting:

0 -> Horizon.

1 -> Zenith.

coord:	Coordinate order in record:

0 -> Northing, Easting, Elevation.

1 -> Easting, Northing, Elevation.

tmode:	Integer designation of the temperature unit:

0 -> Centigrade.

1 -> Fahrenheit.

pmode: Integer designation of the pressure unit:

0 -> mmHg.

1 -> inHg.

2 -> hPa.

sl:     Integer designation of sea level correction on/off:

0 -> off.

1 -> on.

cr:    Integer designation of curvature/refraction correction on/off:

0 -> off.

1 -> on.

scale:	Scale Factor. This is a real number.

crc:	Curvature/refraction coefficient. Normal settings are 	0.132 and 0.200, but anything between 0.5 and 1.5 			inclusive is acceptable.

HAraw:Horizontal Angle Raw Data Setting

0-> Zero to BS

1-> Azimuth

If a parameter is not to be modified, an asterisk (*) must be used as a place holder.  All parameters must be included.

Import ASCII coordinate field order:

Ex:	ns95 /ior fieldlist

fieldlist:		This command specifies the types of field that are imported and their import order. The list of fields is shown below. The fields can be in any order and are represented by typing the following characters  without spaces:

P -> Point number*

I   -> ID

N -> Northing*

E -> Easting*

Z -> Elevation

C -> Code

- (hyphen) -> blank field

- The field list must contain at least those items marked with an asterisk (*).

Import ASCII coordinate file:

Ex:	ns95 /iac jobname filename

jobname:	Name of job to import to. Can include path.

filename:	Name of file to import from. Can include path.

Import Civilsoft coordinate file (Store Ptnum North East (Elev) *Code):

Ex:	ns95 /ics jobname filename

jobname:	Name of job to import to. Can include path.

filename:	Name of file to import from. Can include path.

Export ASCII coordinate file (Ptnum, North, East, Elev, Code).

Ex:	 ns95 /eac jobname filename

jobname:	Name of job to export from. Can include path.

filename:	Name of file to export to. Can include path.

Export File: AutoCAD DXF format

Ex:	ns95 /edx jobname filename

jobname:	Name of job to export from. Can include path.

filename:	Name of file to export to. Can include path.

Export File: Nikon RAW V2.0 (AP-800) data format.

Ex:	ns95 /enr jobname filename

jobname:	Name of job to export from. Can include path.

filename:	Name of file to export to. Can include path.

Export File: SDR2x format.

Ex:	ns95 /esd jobname filename

jobname:	Name of job to export from. Can include path.

filename:	Name of file to export to. Can include path.

Export File: TDS RAW format.

Ex:	ns95 /etd jobname filename

jobname:	Name of job to export from. Can include path.

filename:	Name of file to export to. Can include path.

Export File: Softdesk Field Book .FBK format.

Ex:	ns95 /eso jobname filename

jobname:	Name of job to export from. Can include path.

filename:	Name of file to export to. Can include path.

Export File: Civilsoft coordinate format.

Ex:	ns95 /ecs jobname filename

jobname:	Name of job to export from. Can include path.

filename:	Name of file to export to. Can include path.

Export File: Leica TOPOS format.

Ex:	ns95 /ewt jobname filename

jobname:	Name of job to export from. Can include path.

filename:	Name of file to export to. Can include path.

Create Binary Code or Parameter File:.

Ex:	ns95 /cod textfile codefile

textfile:	Name of text file to convert. Can include path.

codefile:	Name of binary file to create. Can include path.

Return States

NS95 returns the following values to indicate execution status. If the operation was successful, then a return value of “0” occurs.

0	Successful completion.

1	Invalid command line arguments. Either a wrong switch 	was used, or the wrong number of arguments accompanied 	the switch.

2	An incorrect value was used for one or more of the 	arguments.

3	Not enough memory to run program.

4	Not enough disk space.

5	Too many files open.

10	Invalid directory.

11	Invalid file name given.

12	Invalid job name given.

13	Job name is not a valid job.

14	Job name has extension on it. No extension can be given.

15	File does not exist.

16	Job does not exist.

17	File already exists.

18	Job already exists.

- Duplicate records were found. If importing file, existing

record is deleted in job.  If adding coordinate record,

record is ignored.

30	Could not add duplicate record on import.

31	No record found in job.

40	Illegal operation.

52	Code conversion error, see NS95-ERR.TXT for details.

100 	Unknown error.